SVKM's NMIMS — STME Indore

DSM (702MH0C025) — ANSWER BOOKLET — SET 1

Term End Examination — March 2026

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (a)  —  Definitions [5 marks]

(i) DOF: In 2D, a rigid body has 3 DOF (x, y, θ); in 3D, it has 6 DOF (x, y, z + three rotational angles). DOF = total coordinates − number of constraints.  [1.5]

(ii) Rolling without slipping: v_cm = ωR. The contact point has zero velocity relative to the surface. Example: car wheel on road.  [2]

(iii) Instantaneous Center of Rotation (ICR): A point on or off the body about which all points appear to rotate at that instant. Example: contact point of a rolling wheel.  [1.5]

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (b)  —  Projectile Motion [5 marks]

Given: v_x0 = 12 m/s, v_y0 = 16 m/s, g = 9.81 m/s²

h_max = v_y0²/(2g) = 256 / 19.62 = 13.05 m  [1]

t_peak = v_y0/g = 16/9.81 = 1.631 s; Total T = 2 × 1.631 = 3.262 s  [1]

Range R = v_x0 × T = 12 × 3.262 = 39.14 m  [1]

At t = 1 s: v_x = 12 m/s, v_y = 16 − 9.81(1) = 6.19 m/s  [1]

Speed = √(12² + 6.19²) = √(144 + 38.32) = √182.32 = 13.50 m/s  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (c)  —  Atwood Machine [5 marks]

Given: m₁ = 5 kg, m₂ = 3 kg, g = 9.81 m/s²

a = (m₁ − m₂)g / (m₁ + m₂) = (2)(9.81)/8 = 2.453 m/s²  [1.5]

T = 2m₁m₂g / (m₁ + m₂) = 2(5)(3)(9.81)/8 = 36.79 N  [1.5]

v(1.5 s) = a × t = 2.453 × 1.5 = 3.68 m/s  [1]

Check: m₂g = 29.43 N < T = 36.79 N < m₁g = 49.05 N ✓  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (d)  —  Work-Energy on Incline [5 marks]

Given: m = 3 kg, θ = 35°, μ_k = 0.25, s = 4 m, v₀ = 0

N = mg cos 35° = 3(9.81)(0.8192) = 24.11 N  [1]

W_gravity = mgs sin 35° = 3(9.81)(4)(0.5736) = 67.51 J  [1]

W_friction = −μ_k N s = −0.25(24.11)(4) = −24.11 J  [1]

W_net = 67.51 − 24.11 = 43.40 J  [1]

½mv² = W_net → v = √(2 × 43.40 / 3) = √28.93 = 5.38 m/s  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q2  —  Particle Kinetics [20 marks]

Q2 (a)  —  Frictionless Incline [5 marks]

Given: m = 5 kg, θ = 30°, v₀ = 10 m/s up, frictionless

a = −g sin 30° = −9.81 × 0.5 = −4.905 m/s² (decelerating)  [1]

d = v₀² / (2 × 4.905) = 100 / 9.81 = 10.19 m  [2]

t = v₀ / 4.905 = 10 / 4.905 = 2.039 s  [2]

Q2 (b)  —  Block and Tackle [5 marks]

Given: 4 pulleys, η = 0.95 each, m_load = 200 kg, h = 2 m

MA_ideal = 4; η_total = 0.95⁴ = 0.8145  [1]

F_ideal = 200(9.81)/4 = 490.5 N  [1]

F_actual = F_ideal / η_total = 490.5 / 0.8145 = 602.2 N  [1]

W_in = F_actual × (4 × 2) = 602.2 × 8 = 4817.6 J  [1]

W_out = mgh = 200(9.81)(2) = 3924.0 J; Heat = 4817.6 − 3924.0 = 893.6 J  [1]

Q2 (c)  —  Vertical Circular Loop [5 marks]

Given: R = 2 m, m = 1 kg, v_bottom = 12 m/s

v_top_min = √(gR) = √(9.81 × 2) = 4.43 m/s (when N_top = 0)  [1]

v_bottom_min = √(5gR) = √(5 × 9.81 × 2) = √98.1 = 9.90 m/s  [1]

Energy conservation: ½mv²_bottom = ½mv²_top + 2mgR

v²_top = 144 − 4(9.81) = 144 − 39.24 = 104.76; v_top = 10.24 m/s  [1]

N_bottom = mv²/R + mg = 1(144)/2 + 9.81 = 72 + 9.81 = 81.81 N  [2]

N_top = mv²_top/R − mg = 1(104.76)/2 − 9.81 = 52.38 − 9.81 = 42.57 N

Q2 (d)  —  Movable Pulley [5 marks]

Given: m_load = 50 kg, F = 300 N applied

For constant velocity: F = W/2 = 50(9.81)/2 = 245.25 N  [2]

For F = 300 N: Net force = 2F − mg = 600 − 490.5 = 109.5 N  [1.5]

a = 109.5 / 50 = 2.19 m/s²  [1.5]

───────────────────────────────────────────────────────────────────────────────────────────────

Q3  —  Rigid Body & Vibrations [20 marks]

Q3 (a)  —  Rolling Race [5 marks]

For rolling objects: a = g sin θ / (1 + β), where β = I/(mR²)

Sphere: β = 2/5 → a = 9.81 sin 30° / 1.4 = 4.905 / 1.4 = 3.504 m/s²  [1]

Disk: β = 1/2 → a = 4.905 / 1.5 = 3.270 m/s²  [1]

Cylinder (hollow): β = 1 → a = 4.905 / 2 = 2.453 m/s²  [1]

Time (t = √(2s/a)): Sphere: 1.689 s, Disk: 1.749 s, Cylinder: 2.019 s  [1]

Velocity (v = √(2as)): Sphere: 5.92 m/s, Disk: 5.72 m/s, Cylinder: 4.95 m/s  [0.5]

Winner: Solid sphere (lowest β → highest acceleration)  [0.5]

Q3 (b)  —  Physical Pendulum [5 marks]

Given: rod m = 2 kg, L = 1 m, pivot at end

I_O = mL²/3 = 2(1)/3 = 0.667 kg·m²  [1]

d (pivot to CG) = L/2 = 0.5 m  [0.5]

ω_n = √(mgd/I_O) = √(2 × 9.81 × 0.5 / 0.667) = √(14.72) = 3.836 rad/s  [1.5]

T = 2π/ω_n = 2π/3.836 = 1.638 s  [1]

Simple pendulum (same L): T_s = 2π√(1/9.81) = 2.006 s → Physical is 18.4% faster  [1]

Q3 (c)  —  Undamped Spring-Mass [5 marks]

Given: m = 2 kg, k = 200 N/m, x₀ = 0.1 m, v₀ = 0

ω_n = √(k/m) = √(200/2) = √100 = 10 rad/s  [1]

f_n = ω_n/(2π) = 10/6.283 = 1.592 Hz; T = 1/f_n = 0.628 s  [1]

v_max = ω_n × x₀ = 10 × 0.1 = 1.0 m/s  [1.5]

E_total = ½kx₀² = ½(200)(0.01) = 1.0 J  [1.5]

Q3 (d)  —  Spring-Mass-Damper Classification [5 marks]

Given: m = 2 kg, k = 500 N/m, c = 20 N·s/m

ω_n = √(500/2) = √250 = 15.81 rad/s  [1]

c_cr = 2√(km) = 2√(1000) = 63.25 N·s/m  [1]

ζ = c/c_cr = 20/63.25 = 0.316 → Underdamped (ζ < 1)  [1]

ω_d = ω_n √(1 − ζ²) = 15.81 × √(1 − 0.0999) = 15.81 × 0.9487 = 15.00 rad/s  [1]

Amplitude envelope: e^(−ζω_n t) = e^(−5t); For 10%: t = ln(10)/5 = 0.461 s  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q4  —  Rotating Frames [20 marks]

Q4 (a)  —  Transport Theorem [5 marks]

Statement: (dA/dt)_fixed = (dA/dt)_rotating + Ω × A for any vector A  [2]

Velocity relation: v_fixed = v_rel + Ω × r + v_origin

v_rel: velocity measured in rotating frame  [1]

Ω × r: velocity due to rotation of the frame itself  [1]

v_origin: velocity of the rotating frame's origin  [1]

Q4 (b)  —  Acceleration in Rotating Frame [5 marks]

a_fixed = a_rel + 2Ω × v_rel + Ω × (Ω × r) + α × r + a_origin  [2]

Coriolis: 2Ω × v_rel — deflection of moving objects due to frame rotation  [1]

Centripetal: Ω × (Ω × r) — always directed toward the rotation axis  [1]

Euler: α × r — present only when angular velocity is changing  [1]

Q4 (c)  —  Rotating Platform Numerical [5 marks]

Given: Ω = 3.0 k̂ rad/s, r = 1.0 î m, v_rel = (0.5î + 0.8ĵ) m/s, a_rel = (0.2î − 0.1ĵ), α = 0

Coriolis = 2Ω × v_rel = 2(3k̂) × (0.5î + 0.8ĵ) = 6(0.5ĵ − 0.8î) = (−4.8î + 3.0ĵ) m/s²  [1.5]

Using k̂ × î = ĵ and k̂ × ĵ = −î

Centripetal = Ω × (Ω × r) = 3k̂ × (3k̂ × 1.0î) = 3k̂ × 3ĵ = 9(−î) = −9.0î m/s²  [1.5]

Euler = α × r = 0 (since α = 0)

Total: a = (0.2 − 4.8 − 9.0)î + (−0.1 + 3.0)ĵ = (−13.6î + 2.9ĵ) m/s²  [1]

|a| = √(13.6² + 2.9²) = √(184.96 + 8.41) = √193.37 = 13.91 m/s²  [1]

Q4 (d)  —  Coriolis on Earth [5 marks]

Given: latitude λ = 45°N, v = 500 m/s due north, Ω = 7.3 × 10⁻⁵ rad/s, t = 10 s

Vertical component: Ω_v = Ω sin λ = 7.3 × 10⁻⁵ × sin 45° = 5.162 × 10⁻⁵ rad/s  [1]

a_Coriolis = 2Ω_v × v = 2(5.162 × 10⁻⁵)(500) = 0.05162 m/s²  [2]

Deflection d = ½ a_C t² = ½(0.05162)(100) = 2.58 m to the east (right)  [2]

In Northern hemisphere, Coriolis deflects to the right of the velocity direction

───────────────────────────────────────────────────────────────────────────────────────────────

Q5  —  Constitutive Laws & Energy [20 marks]

Q5 (a)  —  Euler-Lagrange Derivation [5 marks]

Hamilton's Principle: δ∫(T − V)dt = 0 over t₁ to t₂  [1]

Lagrangian: L = T − V, where T = kinetic energy, V = potential energy  [1]

Euler-Lagrange: d/dt(∂L/∂q̇) − ∂L/∂q = 0 for each generalized coordinate q  [1]

Generalized coordinates: minimum set of independent variables describing configuration  [1]

Advantage: constraint forces never appear → works naturally with complex systems  [1]

Q5 (b)  —  Newton vs Lagrange Comparison [5 marks]

Newton: Force-based, requires FBD, explicitly handles constraint forces  [1]

Lagrange: Energy-based, uses scalar T and V, constraints handled automatically  [1]

Newton preferred: when forces are needed (e.g., joint reactions)  [1]

Lagrange preferred: multi-DOF, complex constraints, no need for reaction forces  [1]

Simple pendulum: Newton gives mg sin θ = mLα; Lagrange gives same via T = ½mL²θ̇², V = −mgL cos θ  [1]

Q5 (c)  —  Ramp + Rough Surface [5 marks]

Given: m = 4 kg, h = 3 m (frictionless), μ_k = 0.3 (horizontal)

At bottom: mgh = ½mv² → v = √(2gh) = √(2 × 9.81 × 3) = √58.86 = 7.67 m/s  [2]

On horizontal: ½mv² = μ_k mg d → d = v²/(2μ_k g) = 58.86/(2 × 0.3 × 9.81) = 10.0 m  [3]

Q5 (d)  —  Impulse and Ball-Bat Problem [5 marks]

Given: m = 0.15 kg, v₁ = 40 m/s (→), v₂ = 60 m/s (←), Δt = 0.002 s

Taking initial direction as positive: v₂ = −60 m/s

J = m(v₂ − v₁) = 0.15(−60 − 40) = −15 N·s; |J| = 15 N·s  [1.5]

F_avg = |J|/Δt = 15/0.002 = 7500 N  [1.5]

KE_before = ½(0.15)(40²) = 120 J; KE_after = ½(0.15)(60²) = 270 J  [1]

KE NOT conserved — the bat does positive work on the ball (external energy input)  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q6  —  FBD + Difficult Numerical [20 marks]

Q6 (a)  —  Conveyor Belt Transfer [10 marks]

Given: m = 2 kg, v₁ = 1.5 m/s, v₂ = 3.0 m/s, μ_s = 0.35, μ_k = 0.25

FBD [4 marks]:

Weight W = mg = 2(9.81) = 19.62 N (downward)

Normal N = 19.62 N (upward)

Kinetic friction f_k = μ_k N = 0.25(19.62) = 4.905 N (forward, in belt direction)  [2]

FBD shows: block on belt surface; W↓, N↑, f_k→ (direction of belt motion)

Check: package initially slower than belt → belt drags package forward → kinetic friction acts in belt direction ✓  [2]

Calculations [6 marks]:

a = f_k/m = 4.905/2 = 2.453 m/s²  [1.5]

Time to match: t = (v₂ − v₁)/a = 1.5/2.453 = 0.612 s  [1.5]

Package distance: s_pkg = v₁t + ½at² = 1.5(0.612) + ½(2.453)(0.612²) = 0.918 + 0.459 = 1.377 m  [1]

Belt distance: s_belt = v₂ × t = 3.0(0.612) = 1.836 m  [0.5]

Relative sliding = 1.836 − 1.377 = 0.459 m  [0.5]

Heat = f_k × relative sliding = 4.905 × 0.459 = 2.25 J  [1]

Q6 (b)  —  Slider-Crank Mechanism [10 marks]

Given: r = 50 mm, L = 150 mm, n = 3000 RPM, θ = 30°

ω = 2π(3000)/60 = 314.16 rad/s  [2]

sin φ = (r/L) sin θ = (50/150) sin 30° = 0.3333 × 0.5 = 0.1667 → φ = 9.59°  [2]

cos φ = 0.9860  [0.5]

Piston position from crank center: x = r cos θ + L cos φ = 50(0.866) + 150(0.986) = 43.30 + 147.90 = 191.20 mm  [2]

Angular velocity of connecting rod: φ̇ = rω cos θ/(L cos φ) = 50(314.16)(0.866)/(150 × 0.986) = 13599.1/147.9 = 91.95 rad/s  [1.5]

Piston velocity: v = −rω sin θ − Lφ̇ sin φ = −50(314.16)(0.5) − 150(91.95)(0.1667) = −7854 − 2299 = −10153 mm/s = −10.15 m/s  [2]

Negative sign: piston moves toward crank (away from TDC)

───────────────────────────────────────────────────────────────────────────────────────────────

Q7  —  FBD + Difficult Numerical [20 marks]

Q7 (a)  —  Yo-Yo Dynamics [10 marks]

Given: m = 50 g = 0.05 kg, R = 30 mm = 0.03 m, r = 10 mm = 0.01 m (solid disk)

FBD [3 marks]:

Weight W = mg = 0.4905 N (downward at center)

String tension T (upward, applied at spool radius r)

Rolling constraint: a = αr (linear and angular acceleration linked)

Derivation [3 marks]:

I = ½mR² = ½(0.05)(0.03²) = 2.25 × 10⁻⁵ kg·m²  [1]

β = I/(mr²) = R²/(2r²) = 0.0009/(2 × 0.0001) = 4.5  [1]

From ΣF = ma: mg − T = ma; From Στ = Iα: Tr = Iα = I(a/r)

Solving: a = g/(1 + β) = 9.81/5.5 = 1.784 m/s²  [1]

Calculations [4 marks]:

T = m(g − a) = 0.05(9.81 − 1.784) = 0.05(8.026) = 0.401 N  [1]

α = a/r = 1.784/0.01 = 178.4 rad/s²  [1]

KE_rot/KE_total = β/(1 + β) = 4.5/5.5 = 0.818 = 81.8% rotational  [2]

Q7 (b)  —  2-DOF Mass-Spring System [10 marks]

Given: m₁ = 1.0 kg, m₂ = 1.5 kg, k₁ = 100, k₂ = 150, k₃ = 100 N/m

Matrices [3 marks]:

[M] = [[1.0, 0], [0, 1.5]]  [1]

[K] = [[k₁ + k₂, −k₂], [−k₂, k₂ + k₃]] = [[250, −150], [−150, 250]]  [2]

Eigenvalue problem [4 marks]:

det([K] − ω²[M]) = 0

(250 − ω²)(250 − 1.5ω²) − (−150)² = 0  [1]

1.5ω⁴ − 625ω² + 40000 = 0  [1]

ω² = (625 ± √(625² − 4(1.5)(40000))) / (2 × 1.5) = (625 ± √150625) / 3 = (625 ± 388.1) / 3  [1]

ω₁² = 236.9/3 = 78.97 → ω₁ = 8.887 rad/s (f₁ = 1.414 Hz)  [0.5]

ω₂² = 1013.1/3 = 337.7 → ω₂ = 18.38 rad/s (f₂ = 2.925 Hz)  [0.5]

Mode shapes [3 marks]:

Mode 1 (ω₁): Both masses move in-phase (same direction)  [1.5]

Mode 2 (ω₂): Masses move out-of-phase (opposite directions)  [1.5]

───────────────────────────────────────────────────────────────────────────────────────────────