SVKM's NMIMS — STME Indore

DSM (702MH0C025) — ANSWER BOOKLET — SET 3

Term End Examination — March 2026

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (a)  —  Damping Regimes [5 marks]

Underdamped (ζ < 1): Oscillates with exponentially decaying amplitude; ω_d = ω_n√(1 − ζ²)  [1.5]

Critically damped (ζ = 1): Returns to equilibrium fastest without oscillation; x = (A + Bt)e^(−ω_n t)  [1.5]

Overdamped (ζ > 1): Slow return, no oscillation; two real negative roots; sluggish response  [1]

Automotive: slightly underdamped (ζ ≈ 0.2–0.4) — fast return with minimal overshoot (ride quality + control)  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (b)  —  Rocket Kinematics [5 marks]

Given: y(t) = 60t − 10t²

v(t) = dy/dt = 60 − 20t → v₀ = v(0) = 60 m/s  [1]

a = dv/dt = −20 m/s² (constant deceleration)  [1]

At max height: v = 0 → 60 − 20t = 0 → t = 3 s  [1.5]

y_max = 60(3) − 10(9) = 180 − 90 = 90 m  [1.5]

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (c)  —  Atwood 6 kg and 4 kg [5 marks]

Given: m₁ = 6 kg, m₂ = 4 kg, g = 9.81 m/s²

a = (6 − 4)(9.81)/(6 + 4) = 2(9.81)/10 = 1.962 m/s²  [1.5]

T = 2(6)(4)(9.81)/10 = 470.88/10 = 47.09 N  [1.5]

v(1 s) = 1.962 × 1 = 1.962 m/s  [1]

Check: m₂g = 39.24 < T = 47.09 < m₁g = 58.86 ✓  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q1 (d)  —  Coordinate Conversions [5 marks]

(6, 8) → polar: r = √(36 + 64) = √100 = 10; θ = arctan(8/6) = 53.13°  [2]

(ρ = 3, φ = 45°, z = 4) → Cartesian: x = 3 cos 45° = 2.121 m, y = 3 sin 45° = 2.121 m, z = 4 m  [2]

Verification: |r| = √(2.121² + 2.121² + 4²) = √(4.5 + 4.5 + 16) = √25 = 5 m; also √(ρ² + z²) = √(9 + 16) = 5 ✓  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q2  —  Incline & Pulley Systems [20 marks]

Q2 (a)  —  Block Up Rough Incline [5 marks]

Given: m = 5 kg, θ = 30°, μ_k = 0.2, μ_s = 0.3, v₀ = 10 m/s up

Going up (friction opposes = acts down): a = −g(sin 30° + μ_k cos 30°) = −9.81(0.5 + 0.1732) = −6.604 m/s²  [1]

d = v₀²/(2 × 6.604) = 100/13.208 = 7.57 m  [1]

t = v₀/6.604 = 10/6.604 = 1.514 s  [1]

Slide back check: g sin 30° = 4.905 N/kg vs μ_s g cos 30° = 0.3 × 8.496 = 2.549 N/kg  [1]

4.905 > 2.549 → block slides back; a_down = g(sin 30° − μ_k cos 30°) = 9.81 × 0.3268 = 3.206 m/s²  [1]

Q2 (b)  —  Elevator-Counterweight [5 marks]

Given: m_car = 1200 kg, m_cw = 1000 kg, 4 passengers × 75 = 300 kg

Total car side: 1200 + 300 = 1500 kg  [1]

a = (m_car_side − m_cw)g/(m_car_side + m_cw) = (1500 − 1000)(9.81)/(1500 + 1000) = 500(9.81)/2500 = 1.962 m/s² (car descends)  [2]

T = 2(1500)(1000)(9.81)/2500 = 29430000/2500 = 11772 N  [2]

This is an Atwood-type problem. Car side is heavier so it descends.

Q2 (c)  —  Frictionless 30° Incline [5 marks]

Given: m = 1 kg, θ = 30°, frictionless, from rest

N = mg cos 30° = 1(9.81)(0.866) = 8.496 N  [1]

a = g sin 30° = 9.81(0.5) = 4.905 m/s²  [1.5]

v(3) = at = 4.905 × 3 = 14.72 m/s  [1]

s = ½at² = ½(4.905)(9) = 22.07 m  [1.5]

Q2 (d)  —  CNC Trapezoidal/Triangular Profile [5 marks]

Given: d = 250 mm, v_max = 150 mm/s, a = 1000 mm/s²

t_acc = 150/1000 = 0.15 s; d_acc = ½(1000)(0.0225) = 11.25 mm; 2 × d_acc = 22.5 mm < 250 → trapezoidal  [1]

d_cruise = 250 − 22.5 = 227.5 mm; t_cruise = 227.5/150 = 1.517 s  [1.5]

t_total = 0.15 + 1.517 + 0.15 = 1.817 s  [0.5]

For 10 mm: d_min_trap = v_max²/a = 22.5 mm > 10 → triangular  [0.5]

v_peak = √(a × d/2) = √(1000 × 5) = √5000 = 70.71 mm/s; t_total = 2(70.71/1000) = 0.1414 s  [1.5]

───────────────────────────────────────────────────────────────────────────────────────────────

Q3  —  Rigid Body & Energy [20 marks]

Q3 (a)  —  Rolling Disk on 20° Incline [5 marks]

Given: m = 10 kg, R = 0.15 m, θ = 20°, from rest, s = 3 m

Rolling disk: β = 1/2 → a = g sin θ/(1 + 0.5) = 9.81 sin 20°/1.5 = 9.81(0.342)/1.5 = 2.236 m/s²  [1.5]

v = √(2as) = √(2 × 2.236 × 3) = √13.42 = 3.663 m/s  [1]

t = √(2s/a) = √(6/2.236) = √2.683 = 1.638 s  [1]

Frictionless sliding: a = g sin 20° = 3.355 m/s² (50% faster — no rotational KE)  [1.5]

Q3 (b)  —  Pendulum Released from 60° [5 marks]

Given: m = 2 kg, L = 1.5 m, θ₀ = 60°

Height: h = L(1 − cos 60°) = 1.5(1 − 0.5) = 0.75 m  [1]

mgh = ½mv² → v = √(2gh) = √(2 × 9.81 × 0.75) = √14.715 = 3.836 m/s  [2]

At bottom: T − mg = mv²/L → T = mg + mv²/L = 2(9.81) + 2(14.715)/1.5 = 19.62 + 19.62 = 39.24 N  [2]

Q3 (c)  —  Car Braking [5 marks]

Given: m = 1500 kg, v = 100 km/h = 27.78 m/s, F_brake = 8000 N

KE = ½mv² = ½(1500)(771.73) = 578796 J ≈ 578.8 kJ  [1.5]

W = F × d = KE → d = 578796/8000 = 72.35 m  [2]

All KE is dissipated as heat = 578.8 kJ  [1.5]

Q3 (d)  —  Damped Vibration Parameters [5 marks]

Given: m = 2 kg, k = 800 N/m, ζ = 0.2

ω_n = √(k/m) = √(800/2) = √400 = 20 rad/s  [1]

ω_d = ω_n√(1 − ζ²) = 20√(1 − 0.04) = 20 × 0.9798 = 19.60 rad/s  [1]

c_cr = 2mω_n = 2(2)(20) = 80 N·s/m  [1]

c = ζ × c_cr = 0.2 × 80 = 16 N·s/m  [1]

Decay to half: e^(−ζω_n nT_d) = 0.5 → n = ln 2/(ζω_n T_d) = 0.6931/(0.2 × 20 × 2π/19.60) = 0.6931/(4 × 0.3206) = 0.6931/1.2824 = 0.54 → ~1 cycle (amplitude halves in about 1 full oscillation)  [1]

More precisely: per cycle decay = e^(−ζω_n T_d) = e^(−2πζ/√(1−ζ²)) = e^(−1.283) = 0.277, so < 1 cycle needed to halve

───────────────────────────────────────────────────────────────────────────────────────────────

Q4  —  Rigid Body Properties [20 marks]

Q4 (a)  —  MOI & Parallel Axis Theorem [5 marks]

MOI: I = ∫r² dm — measures resistance to angular acceleration  [1]

Parallel Axis: I_O = I_cm + md², where d = distance from CM to new axis  [1]

Rod about center: I = ∫_{−L/2}^{L/2} x² (m/L) dx = (m/L)[x³/3]_{−L/2}^{L/2} = mL²/12  [1.5]

About end (d = L/2): I = mL²/12 + m(L/2)² = mL²/12 + mL²/4 = mL²/3  [1.5]

Q4 (b)  —  Slider-Crank Theory [5 marks]

Components: crank (radius r, angle θ), connecting rod (length L), slider (piston)

Constraint: L sin φ = r sin θ → φ = arcsin[(r/L) sin θ]  [1.5]

Piston position: x = r cos θ + L cos φ  [1]

Asymmetric motion: piston spends more time near TDC than BDC due to rod geometry  [1]

Higher L/r → more symmetric motion, less side thrust, smoother operation  [1.5]

Q4 (c)  —  MOI Calculations [5 marks]

Given: m = 5 kg for each shape

Solid disk (R = 0.2): I = ½mR² = ½(5)(0.04) = 0.1 kg·m²  [1]

Rod (L = 1) about center: I = mL²/12 = 5(1)/12 = 0.417 kg·m²  [1]

Rod about end: I = mL²/3 = 5/3 = 1.667 kg·m²  [1]

Solid sphere (R = 0.2): I = 2mR²/5 = 2(5)(0.04)/5 = 0.08 kg·m²  [1]

PAT check for rod: 0.417 + 5(0.5²) = 0.417 + 1.25 = 1.667 ✓  [1]

Q4 (d)  —  Slider-Crank at 6000 RPM, θ = 90° [5 marks]

Given: r = 50 mm, L = 150 mm, n = 6000 RPM, θ = 90°

ω = 2π(6000)/60 = 628.32 rad/s  [1]

sin φ = (r/L) sin 90° = 50/150 = 0.3333 → φ = 19.47°  [1]

v_piston = −rω sin θ − Lφ̇ sin φ where φ̇ = rω cos θ/(L cos φ) = 50(628.32)(0)/(150 × 0.9428) = 0  [1]

v_piston = −50(628.32)(1) − 0 = −31416 mm/s = −31.42 m/s  [0.5]

At TDC (θ = 0): a_piston ≈ rω²(1 + r/L) = 0.05(628.32²)(1 + 1/3) = 0.05(394826)(1.333) = 26322 m/s² = 2683 g  [1]

2683 g >> 500 g → YES, acceleration exceeds 500 g by a large margin at 6000 RPM  [0.5]

───────────────────────────────────────────────────────────────────────────────────────────────

Q5  —  Lagrangian & Advanced [20 marks]

Q5 (a)  —  Simple Pendulum via Lagrange [5 marks]

Generalized coordinate: θ (angle from vertical)  [0.5]

T = ½m(Lθ̇)² = ½mL²θ̇²; V = −mgL cos θ (datum at pivot)  [1.5]

L = T − V = ½mL²θ̇² + mgL cos θ  [1]

d/dt(∂L/∂θ̇) = mL²θ̈; ∂L/∂θ = −mgL sin θ  [1]

EOM: mL²θ̈ + mgL sin θ = 0 → θ̈ + (g/L) sin θ = 0 — same as Newton (ma_t = −mg sin θ → Lα = −g sin θ)  [1]

Q5 (b)  —  Normal Modes & Modal Analysis [5 marks]

For n-DOF: [M]ẍ + [K]x = 0 → assume x = X e^(iωt)  [1]

Eigenvalue problem: [K]X = ω²[M]X → det([K] − ω²[M]) = 0  [1.5]

Solving gives n natural frequencies ω₁, ω₂, ..., ω_n  [0.5]

Mode shapes: eigenvectors X_i describe the relative motion of each DOF at that frequency  [1]

Physical meaning: at a natural frequency, all parts of the system oscillate in a fixed pattern (mode shape)  [1]

Q5 (c)  —  Simple vs Physical Pendulum [5 marks]

Given: m = 2 kg, L = 1.5 m

Simple: L = ½mL²θ̇² + mgL cos θ; EOM: θ̈ + (g/L) sin θ = 0  [1]

ω_n = √(g/L) = √(9.81/1.5) = √6.54 = 2.557 rad/s; T = 2π/2.557 = 2.457 s  [1.5]

Physical pendulum (rod, same L, pivot at end):

I_O = mL²/3, d = L/2: ω_n = √(mgd/I) = √(2(9.81)(0.75)/(2(2.25)/3)) = √(14.715/1.5) = √9.81 = 3.132 rad/s  [1.5]

T_phys = 2π/3.132 = 2.006 s → physical pendulum is faster (shorter period)  [1]

Q5 (d)  —  ODE Solvers Theory [5 marks]

Forward Euler: y_{n+1} = y_n + h f(t_n, y_n); Order: O(h), 1st order; Simple but unstable for stiff systems  [1.5]

RK2 (Midpoint/Heun): uses slope at midpoint; Order: O(h²); Better accuracy, moderate cost  [1.5]

RK4 (Classical): 4 slope evaluations per step; Order: O(h⁴); Excellent balance of accuracy and efficiency  [1.5]

RK4 preferred: 4th-order accuracy means doubling step size only reduces accuracy by factor 16; no Jacobian needed (unlike implicit methods); widely used in MATLAB ode45, PyBullet, etc.  [0.5]

───────────────────────────────────────────────────────────────────────────────────────────────

Q6  —  FBD + Difficult Numerical [20 marks]

Q6 (a)  —  Rotating Sensor FBD [10 marks]

Given: ω = 2.0 rad/s, r = 0.5 m, ṙ = 0.3 m/s, m = 0.2 kg

FBD in rotating frame [4 marks]:

Weight W = mg = 0.2(9.81) = 1.962 N (downward)  [1]

Centrifugal force: mω²r = 0.2(4)(0.5) = 0.4 N (outward)  [1]

Coriolis force: 2mωṙ = 2(0.2)(2.0)(0.3) = 0.24 N (perpendicular to radial motion)  [1]

Constraint force from track/rail (provides centripetal and tangential forces)  [1]

Acceleration calculations [3 marks]:

Centripetal: a_c = ω²r = (2.0)²(0.5) = 2.0 m/s² (inward)  [1]

Coriolis: a_Cor = 2ωṙ = 2(2.0)(0.3) = 1.2 m/s² (tangential, perpendicular to r)  [1]

|a_total| = √(a_c² + a_Cor²) = √(4.0 + 1.44) = √5.44 = 2.33 m/s²  [1]

Constraint forces [3 marks]:

Radial constraint: F_r = m × a_centripetal = 0.2 × 2.0 = 0.4 N (inward) — maintains circular path  [1.5]

Tangential constraint: F_t = m × a_Cor = 0.2 × 1.2 = 0.24 N — counters Coriolis  [1.5]

Q6 (b)  —  Incline then Horizontal [10 marks]

Given: m = 3 kg, incline 35° μ_k = 0.25 (4 m), horizontal μ_k = 0.3, from rest

On incline [5 marks]:

N₁ = mg cos 35° = 3(9.81)(0.8192) = 24.11 N  [0.5]

h = 4 sin 35° = 4(0.5736) = 2.294 m  [0.5]

W_gravity = mgh = 3(9.81)(2.294) = 67.52 J  [1]

W_friction_incline = −μ_k N₁ s = −0.25(24.11)(4) = −24.11 J  [1]

KE at bottom = 67.52 − 24.11 = 43.41 J; v = √(2 × 43.41/3) = √28.94 = 5.38 m/s  [2]

On horizontal [5 marks]:

N₂ = mg = 29.43 N; f₂ = 0.3(29.43) = 8.83 N  [1]

½mv² = f₂ d → d = 43.41/8.83 = 4.92 m  [2]

Total friction: 24.11 + 8.83 × 4.92 = 24.11 + 43.41 = 67.52 J  [1]

Energy check: All PE (67.52 J) dissipated by friction ✓ (started and ended at rest)  [1]

───────────────────────────────────────────────────────────────────────────────────────────────

Q7  —  FBD + Difficult Numerical [20 marks]

Q7 (a)  —  Conveyor Transfer Energy Analysis [10 marks]

Given: m = 2 kg, v₁ = 1.5 m/s, v₂ = 3.0 m/s, μ_s = 0.35, μ_k = 0.25

FBD [3 marks]:

Weight: W = 2(9.81) = 19.62 N ↓; Normal: N = 19.62 N ↑  [1]

Kinetic friction: f_k = 0.25(19.62) = 4.905 N → (forward, belt pulls package)  [1]

Package is slower than belt: relative motion of package w.r.t. belt is backward → friction acts forward ✓  [1]

Kinematics [4 marks]:

a = f_k/m = 4.905/2 = 2.453 m/s²  [1]

t = Δv/a = 1.5/2.453 = 0.612 s  [1]

ΔKE = ½m(v₂² − v₁²) = ½(2)(9 − 2.25) = 6.75 J  [2]

Energy analysis [3 marks]:

Package displacement: s_p = v₁t + ½at² = 1.5(0.612) + ½(2.453)(0.374) = 0.918 + 0.459 = 1.377 m  [1]

Belt displacement: s_b = v₂t = 3.0(0.612) = 1.836 m  [0.5]

W_friction = f_k × s_b = 4.905 × 1.836 = 9.006 J (total work by friction on belt surface)  [0.5]

Alternative: W_friction_on_package = f_k × s_p = 4.905 × 1.377 = 6.75 J = ΔKE ✓

Efficiency = ΔKE / W_total = 6.75 / 9.006 = 74.9% ≈ 75%  [1]

Q7 (b)  —  Compound Gear Train [10 marks]

Given: Stage 1: 20T → 80T; Stage 2: 15T → 60T; n_in = 2000 RPM, τ_in = 2.0 N·m, η = 96%/stage

Gear ratios [3 marks]:

GR₁ = 80/20 = 4; GR₂ = 60/15 = 4; Total GR = 4 × 4 = 16  [1]

n_out = 2000/16 = 125 RPM = 13.09 rad/s  [1]

η_total = 0.96 × 0.96 = 0.9216  [1]

Torques and power [4 marks]:

τ_out_ideal = τ_in × GR = 2.0 × 16 = 32.0 N·m  [1]

τ_out_actual = τ_out_ideal × η_total = 32.0 × 0.9216 = 29.49 N·m  [1]

P_in = τ_in × ω_in = 2.0 × 2π(2000)/60 = 2.0 × 209.44 = 418.88 W  [1]

P_out = P_in × η_total = 418.88 × 0.9216 = 386.08 W; P_loss = 418.88 − 386.08 = 32.80 W  [1]

Load check [3 marks]:

Required: 25 N·m at 125 RPM → P_load = 25 × 13.09 = 327.25 W  [1]

Available: τ_out = 29.49 N·m > 25 N·m ✓ and P_out = 386.08 W > 327.25 W ✓  [1]

Motor IS sufficient with margin: torque surplus = 4.49 N·m (18%), power surplus = 58.83 W (15.2%)  [1]

───────────────────────────────────────────────────────────────────────────────────────────────