| SVKM’S NMIMS MUKESH PATEL SCHOOL OF TECHNOLOGY MANAGEMENT& ENGINEERING/ SCHOOL OF TECHNOLOGY MANAGEMENT Academic Year: 2025-2026 |  |
| --- | --- |
| Program/s: Bachelor of Technology Stream/s ; Mechatronics | Year: II Semester: IV |
| Subject: Dynamic Systems Modeling | Time: 03 hrs ( to_______) |
| Date: / / 2026 | No. of Pages: 03 |
| Marks: __100_______ |  |
| Re-Examination (2022-2023) |  |

Instructions: Candidates should read carefully the instructions printed on the question paper and on the cover of the Answer Book, which is provided for their use.

1) Question No. _1___ is compulsory.

2) Out of remaining questions, attempt any __4___ questions.

3) In all ___5_ questions to be attempted.

4) All questions carry equal marks.

5) Answer to each new question to be started on a fresh page.

6) Figures in brackets on the right hand side indicate full marks.

7) Assume Suitable data if necessary.

| Q1 |  | Answer briefly: | [20] |
| --- | --- | --- | --- |
| CO-04; BL-02 | a. | State Newton's three laws of motion. For each law, give one example from the field of mechatronics or robotics. Explain how the second law is generalized to rotational motion. |  |
| CO-01,5 ; BL-03 | b. | A uniform rod of mass 3 kg and length 1.2 m is pivoted at a point 0.3 m from one end. Find: (i) the moment of inertia about the center of mass, (ii) the distance from the pivot to the center of mass, and (iii) the moment of inertia about the pivot using the Parallel Axis Theorem. |  |
| CO-05; BL-03 | c. | A 2 kg ball traveling at 8 m/s collides head-on with a stationary 3 kg ball. The coefficient of restitution is e = 0.6. Find: (i) the velocities of both balls after collision, (ii) the kinetic energy lost, and (iii) the impulse experienced by each ball. |  |
| CO-03 ; BL-03 | d. | A 4 kg block is placed on a rough horizontal surface with μ_s = 0.4 and μ_k = 0.35. A horizontal force of 25 N is applied. Find: (i) the normal force and maximum static friction force, (ii) whether the block moves, and (iii) the acceleration of the block. |  |
| Q2 CO-04,5; BL-03 |  | Attempt all four parts. (a) A two-stage gear train has Stage 1 (20-tooth driver meshing with 60-tooth driven) and Stage 2 (18-tooth driver meshing with 90-tooth driven). The input shaft runs at 2400 RPM delivering a torque of 1.2 N·m. Each stage has efficiency 95%. Find: (i) the overall gear ratio, (ii) the output RPM, (iii) the output torque including efficiency losses, and (iv) the total power loss. [5] (b) A solid disk flywheel of mass 50 kg and radius 0.5 m is rotating at 1200 RPM. A braking torque brings it to rest in 30 seconds. Find: (i) the moment of inertia, (ii) the initial angular velocity in rad/s, (iii) the angular deceleration, (iv) the braking torque, and (v) the total kinetic energy absorbed by the brake. [5] (c) A motor drives a load through a worm gear (GR = 40:1, η = 75%). The motor output is 2 kW at 1440 RPM. Find: (i) the motor torque, (ii) the output speed, (iii) the output torque, and (iv) the power lost as heat. [5] (d) A solid cylinder of mass 3 kg and radius 0.1 m is driven by a constant torque of 0.6 N·m against viscous friction (c = 0.2 N·m·s/rad). Find: (i) the moment of inertia, (ii) the steady-state angular velocity, (iii) the time constant, and (iv) the angular velocity after one time constant. [5] | [20] |
| Q3 CO-03; BL-03 |  | Attempt all four parts. (a) A roller-coaster car of mass 800 kg is at the top of a 25 m hill traveling at 5 m/s. It goes down and through a vertical loop of radius 10 m. Using energy conservation, find: (i) the speed at the bottom of the hill, (ii) the speed at the top of the loop (height 20 m from bottom), and (iii) the normal force at the top of the loop. [5] (b) A damped vibration system with mass 5 kg has two successive peaks: x₁ = 25 mm at t₁ = 0.1 s and x₂ = 18 mm at t₂ = 0.38 s. Find: (i) the damped period, (ii) the logarithmic decrement, (iii) the damping ratio, and (iv) the undamped natural frequency and spring stiffness. [5] (c) A 1.5 kg pendulum bob on a 2.0 m string is released from a horizontal position (θ = 90°). Find: (i) the speed at the lowest point, (ii) the tension in the string at the lowest point, and (iii) the height at which the tension equals the weight (if it exists). [5] (d) A 70 kg athlete jumps vertically upward from a crouching position. During the push-off phase (0.3 m), the average leg force is 2500 N. Find: (i) the net upward force, (ii) the velocity at take-off using work-energy, (iii) the maximum jump height, and (iv) the average power during push-off if the phase lasts 0.15 s. [5] | [20] |
| Q4 CO-04,5; BL-04 |  | (a) State and derive the Transport Theorem for the velocity and acceleration of a point observed from a rotating reference frame. Define each acceleration term (Coriolis, centripetal, Euler, relative) with its physical origin. [5] (b) Define the radius of gyration k and its relation to the moment of inertia (I = mk²). State the perpendicular axis theorem and the parallel axis theorem. Give one application of each in engineering design. [5] (c) A centrifuge arm of length r = 2.5 m rotates at 500 RPM. A test sample at the tip of the arm experiences a small vibration in the radial direction with velocity ṙ = 0.05 m/s. Calculate: (i) the centripetal acceleration, (ii) the Coriolis acceleration, (iii) the total acceleration magnitude, and (iv) express the centripetal acceleration in units of g. [5] (d) A compound object consists of a solid disk (m₁ = 4 kg, R = 0.15 m) welded to a uniform rod (m₂ = 2 kg, L = 0.6 m) at the rod's end. The rod extends radially from the edge of the disk. Find the total moment of inertia about the disk center. [5] | [20] |
| Q5 CO-01; BL-04 |  | (a) Compare a simple pendulum and a physical (compound) pendulum. Derive the period of each for small oscillations. Under what condition does a physical pendulum have the same period as a simple pendulum? [5] (b) Explain the concept of normal modes in a 2-DOF vibrating system. What information do the eigenvalues and eigenvectors of the system provide? Describe the physical meaning of the two mode shapes for two masses connected by springs. [5] (c) A U-tube manometer contains a liquid column of total length 0.6 m. When displaced slightly, the liquid oscillates. Derive the equation of motion using the Lagrangian approach (generalized coordinate = displacement x from equilibrium). Find the natural frequency and period of oscillation. [5] (d) A 2-DOF system has masses m₁ = 2 kg and m₂ = 3 kg coupled by springs: Wall — k₁(200 N/m) — m₁ — k₂(300 N/m) — m₂ — k₃(200 N/m) — Wall. Write the [M] and [K] matrices. Solve for the two natural frequencies. Describe the mode shapes. [5] | [20] |
| Q6 |  |  | [20] |
| CO-02; BL-05 | a. | A 5 kg package slides from a chute onto a conveyor belt moving at 4.0 m/s. The package's initial horizontal speed is 1.0 m/s (in the belt direction). Coefficients of friction: μ_s = 0.4, μ_k = 0.3. Draw the complete Free Body Diagram showing all forces during the sliding phase. Determine: (i) the friction force and acceleration, (ii) the time for the package to reach belt speed, (iii) the distances traveled by the package and the belt surface, and (iv) the energy dissipated as heat. | (10) |
| CO-05; BL-05 | b. | A machine of mass 100 kg is mounted on springs with total stiffness k = 40000 N/m and dashpots with total damping c = 800 N·s/m. The machine operates at 1500 RPM and generates a rotating unbalance force of 500 N amplitude. Find: (i) the natural frequency, (ii) the damping ratio, (iii) the frequency ratio, (iv) the steady-state vibration amplitude, and (v) the force transmitted to the foundation (transmissibility). | (10) |
| Q7 |  |  | [20] |
| CO-02; BL-05 | a. | A two-link planar robot arm has link 1 (L₁ = 0.5 m, m₁ = 3 kg) rotating at ω₁ = 2 rad/s with α₁ = 0, and link 2 (L₂ = 0.4 m, m₂ = 2 kg) at ω₂ = 3 rad/s (relative to link 1) with α₂ = 1 rad/s². Both links are in a horizontal plane. At the instant when θ₁ = 0° and θ₂ = 90°, draw the Free Body Diagram of the end-effector mass (1 kg at the tip of link 2). Calculate the absolute velocity and acceleration of the tip using the transport theorem. | (10) |
| CO-03; BL-05 | b. | A Scotch yoke mechanism converts rotary to pure sinusoidal linear motion. The crank radius is r = 40 mm and it rotates at 1800 RPM. The slider has mass 0.5 kg. Find: (i) the maximum displacement, velocity, and acceleration of the slider, (ii) the maximum inertia force on the slider, (iii) the power required to accelerate the slider at the position of maximum acceleration, and (iv) the slider position and velocity when the crank is at θ = 45°. | (10) |