| SVKM'S NMIMS MUKESH PATEL SCHOOL OF TECHNOLOGY MANAGEMENT & ENGINEERING Academic Year: 2025-2026 |  |
| --- | --- |
| Program: B.Tech Stream: Mechatronics | Year: 2nd Semester: IV |
| Subject: Dynamic Systems Modeling (702MH0C023) | Time: 45 Minutes |
| Date: 16-04-2026 | No. of Pages: 1 |
| Marks: 10 | Set: 3 |
| Test-II |  |

Instructions: Candidates should read carefully the instructions.

1) Figures in brackets on the right hand side indicate full marks.

3) Assume Suitable data if necessary.

4) Question 1 is compulsory.

5) Answer any 2 from the remaining questions.

| Q1 |  | Answer briefly: |  |
| --- | --- | --- | --- |
| CO- 1,2; SO- 1,2; BL- 1,2 | a. | (i) Explain the concept of rolling without slipping and state the velocity constraint. (ii) What is a trapezoidal velocity profile in CNC motion planning? Sketch it qualitatively. (iii) Define logarithmic decrement in vibration analysis and state its physical meaning. (iv) State the relationship between mechanical advantage and the number of supporting rope segments in a block-and-tackle system. | [2] |
| CO- 1,2; SO- 1,2; BL- 2,3 | b. | (i) A platform rotates at Ω = 3 rad/s. A particle moves radially outward at v_rel = 2 m/s. Calculate the magnitude of the Coriolis acceleration. (ii) Write the equation of motion for an undamped spring-mass system and identify each term. (iii) Define mechanical advantage of a pulley system. State its ideal value for a single movable pulley. (iv) Explain why fictitious forces (pseudo-forces) appear in a rotating (non-inertial) reference frame. | [2] |
| Q2 CO- 2; SO- 2; BL- 3 |  | Two masses m₁ = 6 kg and m₂ = 4 kg are connected by a light, inextensible string passing over a massless, frictionless pulley. The system is released from rest. Take g = 9.81 m/s². (a) Draw the free-body diagram for each mass and write the equation of motion for m₁. (b) Write the equation of motion for m₂. (c) Find the acceleration of the system. (d) Calculate the tension in the string. (e) Find the velocity of the masses after 2 seconds. (f) Calculate the distance traveled by each mass in 2 seconds. | [3] |
| Q3 CO- 2; SO- 2; BL- 3,4 |  | A slider-crank mechanism has crank radius r = 50 mm, connecting rod length L = 150 mm, and operates at 3000 RPM. Analyze the mechanism at crank angle θ = 30°. (a) Convert the crank speed from RPM to rad/s. (b) Calculate the connecting rod angle φ using the kinematic relation r sin θ = L sin φ. (c) Determine the piston displacement from top dead center (TDC). (d) Find the piston velocity. (e) Identify the crank angles at which the piston velocity is zero. (f) State the type of motion executed by the piston and name one engineering application. | [3] |
| Q4 CO- 2,3; SO- 2,3; BL- 4,5 |  | A yo-yo of mass m = 0.05 kg has outer radius R = 0.03 m, axle radius r = 0.01 m, and moment of inertia I = 2.25 × 10⁻⁵ kg·m² (modeled as a solid disk of radius R). The string unwinds from the axle. Take g = 9.81 m/s². (a) Write the translational equation of motion for the yo-yo (vertical forces). (b) Write the rotational equation of motion about the center of mass. (c) State the constraint relating linear acceleration a and angular acceleration α. (d) Solve for the linear acceleration of the yo-yo. (e) Calculate the tension in the string. (f) Express the acceleration as a fraction of g and explain why the yo-yo descends slower than free fall. | [3] |