Mukesh Patel School of Technology Management & Engineering

B.Tech Mechatronics Engineering, Semester IV

Mid Semester Examination – II — MODEL ANSWERS

| Course: | Dynamic Systems Modeling (702MH0C023) | Date: | 16-04-2026 |
| --- | --- | --- | --- |
| Duration: | 45 minutes | Total Marks: | 10 |
| Set: | 1 | Program: | B.Tech Mechatronics Engg. |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

Q1(a) [2 Marks]    Bloom: BL 1–2 (Remember / Understand)  |  CO1, CO2

Answer the following:

(a)  Define the Coriolis acceleration and state when it becomes significant in engineering analysis.  [0.5]

Ans: The Coriolis acceleration is the term 2Ω × v_rel that arises when an object moves with velocity v_rel in a reference frame rotating at angular velocity Ω. It becomes significant when the product Ω·v_rel is comparable to other acceleration terms, e.g. in large-scale meteorology, long-range ballistics, or fast-rotating machinery.

(b)  What is an Atwood machine? State the underlying principle on which it operates.  [0.5]

Ans: An Atwood machine consists of two masses connected by a light inextensible string passing over a frictionless pulley. It works on Newton's second law applied to a constrained two-body system: the heavier mass accelerates downward while the lighter mass accelerates upward, with a common acceleration determined by the mass difference.

(c)  Define the moment of inertia of a rigid body and state its SI unit.  [0.5]

Ans: The moment of inertia (I) of a rigid body about a given axis is the sum (or integral) of the products of each mass element and the square of its distance from the axis: I = Σ mᵢrᵢ² (discrete) or I = ∫ r² dm (continuous). SI unit: kg·m².

(d)  Differentiate between undamped and damped free vibrations.  [0.5]

Ans: Undamped free vibration: the system oscillates indefinitely at its natural frequency ω_n with constant amplitude (no energy loss). Damped free vibration: a dissipative element (damper, friction) causes the amplitude to decay over time; the oscillation frequency ω_d is slightly lower than ω_n, and the system eventually comes to rest.

Q1(b) [2 Marks]    Bloom: BL 2–3 (Understand / Apply)  |  CO1, CO2

(a)  A solid disk of mass 5 kg and radius 0.2 m rolls without slipping. Calculate its moment of inertia about the contact point using the parallel axis theorem.  [0.5]

Ans: I_cm = (1/2)mR² = 0.5 × 5 × 0.2² = 0.10 kg·m². By parallel axis theorem: I_contact = I_cm + mR² = 0.10 + 5 × 0.04 = 0.10 + 0.20 = 0.30 kg·m².

(b)  Explain why a solid sphere reaches the bottom of an incline before a hollow cylinder of the same mass and radius.  [0.5]

Ans: The rolling acceleration is a = g sinθ / (1 + β), where β = I/(mR²). Solid sphere: β = 2/5 = 0.4, hollow cylinder: β = 1.0. Since the sphere has a smaller β, it has a larger acceleration and reaches the bottom first. The hollow cylinder puts more energy into rotation.

(c)  In an Atwood machine with masses 5 kg and 3 kg, state which mass accelerates downward and why.  [0.5]

Ans: The 5 kg mass accelerates downward because it is heavier. In an Atwood machine, the net downward force is (m₁ − m₂)g = (5 − 3) × 9.81 = 19.62 N, and this causes the heavier mass to descend while the lighter mass rises.

(d)  State two assumptions commonly made when analyzing an ideal pulley system.  [0.5]

Ans: (1) The pulley is massless and frictionless (no rotational inertia, no axle friction). (2) The string/rope is massless, inextensible, and has uniform tension throughout its length.

Solve any TWO from Q2, Q3, Q4  [3 Marks each = 6 Marks]

Q2. [3 Marks]    Bloom: BL 3 (Apply)  |  CO2

Two masses m₁ = 5 kg and m₂ = 3 kg are connected by a light, inextensible string passing over a massless, frictionless pulley. The system is released from rest. Take g = 9.81 m/s².

(a)  Draw the free-body diagram for each mass and write the equation of motion for m₁.  [0.5]

Ans: FBD for m₁: Weight m₁g = 5×9.81 = 49.05 N downward, Tension T upward.

Equation of motion (taking downward positive for m₁): m₁g − T = m₁a

→ 49.05 − T = 5a

(b)  Write the equation of motion for m₂.  [0.5]

Ans: FBD for m₂: Weight m₂g = 3×9.81 = 29.43 N downward, Tension T upward.

Equation of motion (taking upward positive for m₂): T − m₂g = m₂a

→ T − 29.43 = 3a

(c)  Find the acceleration of the system.  [0.5]

Ans: Adding both equations: m₁g − m₂g = (m₁ + m₂)a

a = (m₁ − m₂)g / (m₁ + m₂) = (5 − 3) × 9.81 / (5 + 3)

a = 19.62 / 8 = 2.453 m/s²

(d)  Calculate the tension in the string.  [0.5]

Ans: From equation for m₂: T = m₂(g + a) = 3 × (9.81 + 2.453)

T = 3 × 12.263 = 36.79 N

Check via m₁: T = m₁(g − a) = 5 × 7.357 = 36.79 N ✓

(e)  Find the velocity of the masses after 2 seconds.  [0.5]

Ans: v = u + at = 0 + 2.453 × 2 = 4.905 m/s

(f)  Calculate the distance traveled by each mass in 2 seconds.  [0.5]

Ans: s = ½at² = 0.5 × 2.453 × 2² = 0.5 × 2.453 × 4 = 4.905 m

Q3. [3 Marks]    Bloom: BL 3–4 (Apply / Analyze)  |  CO2

A platform rotates at constant angular velocity Ω = 3 rad/s about a vertical axis. An object at r = 2 m from the center moves radially outward at v_rel = 1.5 m/s with no radial acceleration (r̈ = 0). The angular acceleration of the platform is zero (α = 0).

(a)  Calculate the centripetal acceleration magnitude.  [0.5]

Ans: a_centripetal = Ω²r = 3.0² × 2.0 = 18.0 m/s²

(b)  Calculate the Coriolis acceleration magnitude.  [0.5]

Ans: a_Coriolis = 2Ωv_rel = 2 × 3.0 × 1.5 = 9.0 m/s²

(c)  State the direction of the Coriolis acceleration relative to the radial velocity.  [0.5]

Ans: The Coriolis acceleration is perpendicular to v_rel (the radial direction), directed tangentially in the sense opposite to the platform rotation (by the right-hand rule for 2Ω × v_rel).

(d)  Find the total acceleration magnitude in the rotating frame.  [0.5]

Ans: |a_total| = √(a_cent² + a_Cor²) = √(18.0² + 9.0²) = √(324 + 81) = √405 = 20.12 m/s²

(e)  Determine the tangential velocity of the object as observed from the fixed (inertial) frame.  [0.5]

Ans: v_tangential = Ωr = 3.0 × 2.0 = 6.0 m/s

(f)  If the object continues at the same radial speed, find the centripetal acceleration when r = 4 m.  [0.5]

Ans: At r = 4 m: a_centripetal = Ω²r = 3.0² × 4 = 36.0 m/s²

Q4. [3 Marks]    Bloom: BL 4–5 (Analyze / Evaluate)  |  CO2, CO3

A uniform rod of mass m = 2 kg and length L = 1 m is pivoted at one end and oscillates as a physical pendulum under gravity. Take g = 9.81 m/s².

(a)  Calculate the moment of inertia of the rod about the pivot using the parallel axis theorem.  [0.5]

Ans: I_cm = (1/12)mL² = (1/12) × 2.0 × 1.0² = 0.1667 kg·m²

I_pivot = I_cm + m(L/2)² = 0.1667 + 2.0 × 0.25 = 0.1667 + 0.5000 = 0.6667 kg·m²

(Or directly: I_pivot = (1/3)mL² = 0.6667 kg·m²)

(b)  Find the distance of the center of mass from the pivot.  [0.5]

Ans: d = L/2 = 1.0/2 = 0.5 m

(c)  Write the equation of motion for small-angle oscillations.  [0.5]

Ans: Restoring torque: τ = −mgd sinθ ≈ −mgdθ (small angle)

Equation: I_pivot θ̈ + mgd θ = 0  →  θ̈ + (mgd/I_pivot) θ = 0

(d)  Determine the natural frequency ω_n.  [0.5]

Ans: ω_n = √(mgd / I_pivot) = √(2.0 × 9.81 × 0.5 / 0.6667)

= √(9.810 / 0.6667) = √14.715 = 3.836 rad/s

(e)  Calculate the time period of oscillation.  [0.5]

Ans: T = 2π / ω_n = 2π / 3.836 = 1.638 s

(f)  Compare this period with that of a simple pendulum of the same length (L = 1 m).  [0.5]

Ans: Simple pendulum (same length L = 1.0 m): T_simple = 2π√(L/g) = 2π√(1.0/9.81) = 2.006 s.

Physical pendulum period (1.638 s) < Simple pendulum period (2.006 s) because the physical pendulum's mass is distributed closer to the pivot, reducing the effective pendulum length.

— End of Model Answers —