Mukesh Patel School of Technology Management & Engineering

B.Tech Mechatronics Engineering, Semester IV

Mid Semester Examination – II — MODEL ANSWERS

| Course: | Dynamic Systems Modeling (702MH0C023) | Date: | 16-04-2026 |
| --- | --- | --- | --- |
| Duration: | 45 minutes | Total Marks: | 10 |
| Set: | 2 | Program: | B.Tech Mechatronics Engg. |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

Q1(a) [2 Marks]    Bloom: BL 1–2 (Remember / Understand)  |  CO1, CO2

Answer the following:

(a)  State the parallel axis theorem and give its mathematical expression.  [0.5]

Ans: The parallel axis theorem states that the moment of inertia about any axis parallel to an axis through the center of mass is: I = I_cm + md², where I_cm = MOI about the CM axis, m = total mass, d = perpendicular distance between the two axes. This is the minimum-shift property: I is always smallest about the CM.

(b)  Define the term 'gear ratio' and explain its physical significance.  [0.5]

Ans: The gear ratio (GR) is the ratio of the number of teeth (or radius) of the driven gear to the driving gear: GR = N_driven / N_driver. Physically, it determines the speed and torque transformation: output speed = input speed / GR, and output torque = input torque × GR (ignoring losses). A GR > 1 reduces speed but amplifies torque.

(c)  What is a slider-crank mechanism? Name two practical applications.  [0.5]

Ans: A slider-crank mechanism converts rotary motion (crank) into reciprocating linear motion (slider/piston) via a connecting rod. The kinematic constraint is r sin θ = L sin φ, relating the crank angle θ to the connecting rod angle φ. Applications: (1) Internal combustion engines (piston–crankshaft), (2) Reciprocating compressors / pumps.

(d)  Define critical damping and state the mathematical condition for it.  [0.5]

Ans: Critical damping is the boundary between oscillatory (underdamped) and non-oscillatory (overdamped) response. Mathematical condition: damping ratio ζ = 1, i.e. c = c_cr = 2√(km), where c = damping coefficient, k = stiffness, m = mass. The system returns to equilibrium in the shortest time without overshooting.

Q1(b) [2 Marks]    Bloom: BL 2–3 (Understand / Apply)  |  CO1, CO2

(a)  A gear train has an input speed of 100 RPM and a gear ratio of 15 : 1 (speed reduction). Calculate the output shaft speed.  [0.5]

Ans: Output speed = Input speed / GR = 100 / 15 = 6.67 RPM.

(b)  Explain the physical meaning of the damping ratio ζ and what happens when ζ = 1.  [0.5]

Ans: The damping ratio ζ = c / c_cr is the ratio of actual damping to critical damping. It determines the system's response type: ζ < 1 → underdamped (oscillates with decaying amplitude), ζ = 1 → critically damped (fastest return to equilibrium without overshoot), ζ > 1 → overdamped (slow, non-oscillatory return).

(c)  A CNC axis must traverse 200 mm with v_max = 100 mm/s and a_max = 500 mm/s². Estimate the total traverse time using a trapezoidal profile.  [0.5]

Ans: t_accel = v_max / a_max = 100/500 = 0.2 s. d_accel = v_max² / (2 a_max) = 10000/1000 = 10 mm. d_cruise = 200 − 2 × 10 = 180 mm. t_cruise = 180/100 = 1.8 s. Total time = 0.2 + 1.8 + 0.2 = 2.2 s.

(d)  State the condition for a block to remain stationary on an inclined plane with friction.  [0.5]

Ans: The block remains stationary when the component of gravity along the incline does not exceed the maximum static friction: mg sinθ ≤ μ_s mg cosθ, i.e. tanθ ≤ μ_s, where μ_s is the coefficient of static friction.

Solve any TWO from Q2, Q3, Q4  [3 Marks each = 6 Marks]

Q2. [3 Marks]    Bloom: BL 3 (Apply)  |  CO2

Two masses m₁ = 8 kg and m₂ = 5 kg are connected by a light, inextensible string passing over a massless, frictionless pulley. The system is released from rest. Take g = 9.81 m/s².

(a)  Draw the free-body diagram for each mass and write the equation of motion for m₁.  [0.5]

Ans: FBD for m₁: Weight m₁g = 8×9.81 = 78.48 N downward, Tension T upward.

Equation of motion (taking downward positive for m₁): m₁g − T = m₁a

→ 78.48 − T = 8a

(b)  Write the equation of motion for m₂.  [0.5]

Ans: FBD for m₂: Weight m₂g = 5×9.81 = 49.05 N downward, Tension T upward.

Equation of motion (taking upward positive for m₂): T − m₂g = m₂a

→ T − 49.05 = 5a

(c)  Find the acceleration of the system.  [0.5]

Ans: Adding both equations: m₁g − m₂g = (m₁ + m₂)a

a = (m₁ − m₂)g / (m₁ + m₂) = (8 − 5) × 9.81 / (8 + 5)

a = 29.43 / 13 = 2.264 m/s²

(d)  Calculate the tension in the string.  [0.5]

Ans: From equation for m₂: T = m₂(g + a) = 5 × (9.81 + 2.264)

T = 5 × 12.074 = 60.37 N

Check via m₁: T = m₁(g − a) = 8 × 7.546 = 60.37 N ✓

(e)  Find the velocity of the masses after 3 seconds.  [0.5]

Ans: v = u + at = 0 + 2.264 × 3 = 6.792 m/s

(f)  Calculate the distance traveled by each mass in 3 seconds.  [0.5]

Ans: s = ½at² = 0.5 × 2.264 × 3² = 0.5 × 2.264 × 9 = 10.187 m

Q3. [3 Marks]    Bloom: BL 3–4 (Apply / Analyze)  |  CO2

A block of mass m = 5 kg is placed on a rough inclined plane at angle θ = 30° to the horizontal. The coefficient of kinetic friction is μ_k = 0.2. The block is released from rest. Take g = 9.81 m/s².

(a)  Draw the free-body diagram showing all forces on the block.  [0.5]

Ans: Forces: (1) Weight W = mg = 49.05 N downward, (2) Normal force N perpendicular to incline (upward from surface), (3) Kinetic friction f = μ_k N opposing motion (up the incline).

(b)  Calculate the normal reaction force N.  [0.5]

Ans: N = mg cos θ = 5.0 × 9.81 × cos 30° = 49.05 × 0.8660 = 42.48 N

(c)  Calculate the kinetic friction force f.  [0.5]

Ans: f = μ_k × N = 0.2 × 42.48 = 8.50 N

(d)  Determine the net force along the incline and find the acceleration.  [0.5]

Ans: F_net = mg sinθ − f = 24.52 − 8.50 = 16.03 N

a = F_net / m = 16.03 / 5.0 = 3.21 m/s²

(e)  Find the velocity of the block after sliding 5 m from rest.  [0.5]

Ans: v = √(2as) = √(2 × 3.21 × 5.0) = √32.06 = 5.66 m/s

(f)  Calculate the time taken to slide 5 m from rest.  [0.5]

Ans: t = √(2s/a) = √(2 × 5.0 / 3.21) = √3.12 = 1.77 s

Q4. [3 Marks]    Bloom: BL 4–5 (Analyze / Evaluate)  |  CO2, CO3

A mass m = 2 kg is attached to a spring of stiffness k = 200 N/m. The system is displaced x₀ = 0.1 m from equilibrium and released from rest (v₀ = 0).

(a)  Write the equation of motion for the undamped system.  [0.5]

Ans: Equation of motion: mẍ + kx = 0  →  2.0ẍ + 200.0x = 0

(b)  Calculate the natural frequency ω_n and the time period T.  [0.5]

Ans: ω_n = √(k/m) = √(200.0/2.0) = √100 = 10.00 rad/s

T = 2π/ω_n = 2π/10.00 = 0.6283 s ≈ 0.628 s

(c)  Write the displacement as a function of time x(t) for the given initial conditions.  [0.5]

Ans: With x(0) = 0.1 m, ẋ(0) = 0:

x(t) = x₀ cos(ω_n t) = 0.1 cos(10t) m

(Amplitude 0.1 m, purely cosinusoidal, no decay)

(d)  A viscous damper with c = 4 N·s/m is now added. Calculate the critical damping coefficient c_cr and the damping ratio ζ.  [0.5]

Ans: c_cr = 2√(km) = 2√(200.0 × 2.0) = 2√400 = 2 × 20.0 = 40.0 N·s/m

ζ = c / c_cr = 4.0 / 40.0 = 0.10

(e)  State whether the damped system is underdamped, critically damped, or overdamped. Calculate the damped natural frequency ω_d.  [0.5]

Ans: Since ζ = 0.10 < 1 → Underdamped (oscillatory with decay).

ω_d = ω_n√(1 − ζ²) = 10.00√(1 − 0.1²) = 10.00 × 0.9950 = 9.95 rad/s

(f)  Calculate the logarithmic decrement δ for the damped system.  [0.5]

Ans: δ = 2πζ / √(1 − ζ²) = 2π × 0.1 / √(1 − 0.1²)

= 0.6283 / 0.9950 = 0.6315

This means each successive peak is e^(−δ) = e^(−0.6315) = 0.5318 times the previous peak.

— End of Model Answers —