Mukesh Patel School of Technology Management & Engineering

B.Tech Mechatronics Engineering, Semester IV

Re-Examination — MODEL ANSWERS — Set 1

| Course: | Dynamic Systems Modeling (702MH0C023) | Date: |  |
| --- | --- | --- | --- |
| Duration: | 45 minutes | Total Marks: | 10 |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

Q1(a) [2 Marks]    Bloom: BL 1–2 (Remember / Understand)  |  CO1

Answer the following:

(a)  Write the expressions for velocity in polar coordinates (v_r and v_θ components) and state when polar coordinates are preferred over Cartesian.  [0.5]

Ans: In polar coordinates:

v_r = ṙ  (radial component)

v_θ = rθ̇  (transverse component)

→ v⃗ = ṙ ê_r + rθ̇ ê_θ

Polar coordinates are preferred when the motion has radial symmetry or when dealing with central-force problems (e.g. planetary motion, pendulums), because the constraint is naturally expressed in r and θ.

(b)  Define the Lagrangian L of a mechanical system and state the Euler-Lagrange equation in terms of a generalized coordinate q.  [0.5]

Ans: The Lagrangian is defined as L = T − V, where T = kinetic energy and V = potential energy.

The Euler-Lagrange equation is:

d/dt (∂L/∂q̇) − ∂L/∂q = 0

This yields the equation(s) of motion for the system in terms of the generalized coordinate q.

(c)  State the transport theorem for a vector quantity as observed from a fixed (inertial) frame and a rotating frame. Identify the correction term.  [0.5]

Ans: The transport theorem states:

(dA⃗/dt)_fixed = (dA⃗/dt)_rotating + Ω⃗ × A⃗

where Ω⃗ is the angular velocity of the rotating frame.

The correction term Ω⃗ × A⃗ accounts for the rotation of the basis vectors themselves as seen from the fixed frame.

(d)  Define the damping ratio ζ and state the three response regimes (underdamped, critically damped, overdamped) with their conditions.  [0.5]

Ans: The damping ratio ζ = c / c_cr = c / (2√(km)), where c = damping coefficient, k = stiffness, m = mass.

Three regimes:

ζ < 1 → Underdamped: oscillatory with exponentially decaying amplitude.

ζ = 1 → Critically damped: fastest return to equilibrium without overshoot.

ζ > 1 → Overdamped: slow, non-oscillatory return to equilibrium.

Q1(b) [2 Marks]    Bloom: BL 2–3 (Understand / Apply)  |  CO1, CO2

(a)  A particle moves in a circle of radius r = 2 m at constant angular speed θ̇ = 3 rad/s. Calculate the radial (centripetal) acceleration a_r and the transverse acceleration a_θ.  [0.5]

Ans: For circular motion at constant angular speed (ṙ = 0, r̈ = 0, θ̈ = 0):

a_r = r̈ − rθ̇² = 0 − 2 × 3² = −18 m/s²  (directed radially inward)

a_θ = rθ̈ + 2ṙθ̇ = 0 + 0 = 0 m/s²

The magnitude of centripetal acceleration is 18 m/s².

(b)  Write the kinetic energy T and potential energy V for a simple pendulum of mass m and length L, using θ as the generalized coordinate.  [0.5]

Ans: For a simple pendulum with angle θ:

T = ½ m (Lθ̇)² = ½ m L² θ̇²

V = −mgL cos θ  (taking the pivot as the datum)

or equivalently V = mgL(1 − cos θ) with datum at the lowest point.

The Lagrangian is L = T − V = ½mL²θ̇² + mgL cos θ.

(c)  Define a holonomic constraint and state how the number of degrees of freedom (DOF) is determined for a system of N particles with k holonomic constraints.  [0.5]

Ans: A holonomic constraint is one that can be expressed as an equation relating only coordinates (and possibly time), not velocities: f(q₁, q₂, …, t) = 0.

DOF = 3N − k, where N = number of particles, k = number of independent holonomic constraints.

Example: A bead on a wire (k = 2 of 3 coordinates constrained → 1 DOF).

(d)  A spring-mass system has m = 4 kg and k = 256 N/m. Calculate the natural frequency ω_n (rad/s) and the time period T of free oscillation.  [0.5]

Ans: ω_n = √(k/m) = √(256/4) = √64 = 8 rad/s.

T = 2π/ω_n = 2π/8 = π/4 ≈ 0.785 s.

Solve any TWO from Q2, Q3, Q4  [3 Marks each = 6 Marks]

Q2. [3 Marks]    Bloom: BL 3 (Apply)  |  CO1, CO2

A simple pendulum consists of a point mass m = 1 kg attached to a massless, inextensible string of length L = 1 m. The bob swings in a vertical plane under gravity (g = 9.81 m/s²). Using the angle θ from the vertical as the generalized coordinate, analyze the system using the Lagrangian method.
Use the following relations: Lagrangian L = T − V; Euler-Lagrange equation: d/dt(∂L/∂q̇) − ∂L/∂q = 0; for small angles: sin θ ≈ θ.

(a)  State the number of degrees of freedom of this system and identify the generalized coordinate.  [0.5]

Ans: The pendulum has 1 degree of freedom (DOF = 1).

Generalized coordinate: θ — the angle of the string measured from the vertical.

(b)  Write the position of the mass in Cartesian coordinates (x, y) in terms of θ and L.  [0.5]

Ans: Taking the pivot as the origin, with x horizontal and y vertically downward:

x = L sin θ = 1.0 sin θ

y = L cos θ = 1.0 cos θ

Differentiating: ẋ = Lθ̇ cos θ,  ẏ = −Lθ̇ sin θ

(c)  Derive the kinetic energy T in terms of θ, θ̇, m, and L.  [0.5]

Ans: v² = ẋ² + ẏ² = L²θ̇²cos²θ + L²θ̇²sin²θ = L²θ̇²

T = ½mv² = ½ × 1.0 × 1.0² × θ̇² = ½mL²θ̇²

T = ½(1.0)(1.0²)θ̇² = 0.5 θ̇²  J

(d)  Write the potential energy V in terms of θ, m, g, and L.  [0.5]

Ans: Taking the pivot as the datum (y positive downward):

V = −mgy = −mgL cos θ = −1.0 × 9.81 × 1.0 × cos θ

V = −9.81 cos θ  J

(e)  Form the Lagrangian L = T − V and apply the Euler-Lagrange equation to derive the equation of motion.  [0.5]

Ans: L = T − V = ½mL²θ̇² + mgL cos θ

Euler-Lagrange: d/dt(∂L/∂θ̇) − ∂L/∂θ = 0

∂L/∂θ̇ = mL²θ̇ = 1.0 × 1.0² × θ̇ = 1.0 θ̇

d/dt(mL²θ̇) = mL²θ̈ = 1.0 θ̈

∂L/∂θ = −mgL sin θ = −9.81 sin θ

EOM: 1.0 θ̈ + 9.81 sin θ = 0

Simplifying (divide by mL²): θ̈ + (g/L) sin θ = 0

→ θ̈ + (9.81/1.0) sin θ = 0  →  θ̈ + 9.81 sin θ = 0

(f)  For small oscillations (sin θ ≈ θ), determine the natural frequency of the pendulum.  [0.5]

Ans: For small angles, sin θ ≈ θ → θ̈ + (g/L)θ = 0

This is SHM with ω_n² = g/L = 9.81/1.0 = 9.81

ω_n = √(9.81) = 3.132 rad/s

Time period T = 2π/ω_n = 2π/3.132 = 2.006 s

Q3. [3 Marks]    Bloom: BL 3–4 (Apply / Analyze)  |  CO2

A horizontal platform rotates at a constant angular velocity Ω = 4 rad/s about a vertical axis. An object at a radial distance r = 1.5 m from the center moves radially outward at a constant speed v_rel = 2 m/s (ṙ = 2 m/s, r̈ = 0). The angular acceleration of the platform is zero (α = 0).

(a)  Write the general expression relating the acceleration observed from the fixed (inertial) frame to quantities measured in the rotating frame. Identify each term.  [0.5]

Ans: The fixed-frame (inertial) acceleration, expressed in terms of rotating-frame quantities, is:

a⃗_fixed = a⃗_rel + 2Ω⃗ × v⃗_rel + α⃗ × r⃗ + Ω⃗ × (Ω⃗ × r⃗)

where:

a⃗_rel = acceleration of the object in the rotating frame (r̈ = 0 here),

2Ω⃗ × v⃗_rel = Coriolis acceleration,

α⃗ × r⃗ = Euler (angular acceleration) term (zero here since α = 0),

Ω⃗ × (Ω⃗ × r⃗) = centripetal acceleration (directed radially inward).

(b)  Calculate the centripetal acceleration magnitude at r = 1.5 m.  [0.5]

Ans: a_centripetal = Ω²r = 4.0² × 1.5 = 16 × 1.5 = 24.0 m/s²

Direction: radially inward (toward the axis of rotation).

(c)  Calculate the Coriolis acceleration magnitude.  [0.5]

Ans: a_Coriolis = 2Ωv_rel = 2 × 4.0 × 2.0 = 16.0 m/s²

(d)  State the direction of the Coriolis acceleration relative to the radial velocity.  [0.5]

Ans: By the right-hand rule for 2Ω⃗ × v⃗_rel (Ω along vertical, v_rel radially outward): the Coriolis acceleration is perpendicular to v_rel, directed tangentially in the direction opposite to the platform's rotation.

(e)  Calculate the total acceleration magnitude of the object as observed from the fixed (inertial) frame.  [0.5]

Ans: |a_total| = √(a_cent² + a_Cor²) = √(24.0² + 16.0²)

= √(576 + 256) = √832 = 28.84 m/s²

(f)  If the object continues at the same radial speed, find the centripetal acceleration when r = 3 m and comment on how it changes.  [0.5]

Ans: At r = 3 m: a_centripetal = Ω²r = 4.0² × 3 = 48.0 m/s²

Compared to 24.0 m/s² at r = 1.5 m, the centripetal acceleration has doubled. It increases linearly with radial distance (a_cent ∝ r), while the Coriolis acceleration (16.0 m/s²) remains unchanged since v_rel is constant.

Q4. [3 Marks]    Bloom: BL 4–5 (Analyze / Evaluate)  |  CO2, CO3

A mass m = 3 kg is attached to a spring of stiffness k = 300 N/m and a viscous damper with damping coefficient c = 12 N·s/m. The system undergoes free vibration after being displaced 0.05 m from equilibrium and released from rest. Take the equilibrium position as the origin.

(a)  Write the equation of motion for this damped spring-mass system.  [0.5]

Ans: The equation of motion for a damped SDOF system:

mẍ + cẋ + kx = 0

3.0ẍ + 12.0ẋ + 300.0x = 0

Dividing by m: ẍ + 4.0ẋ + 100.0x = 0

Or: ẍ + 2ζω_n ẋ + ω_n² x = 0

(b)  Calculate the natural frequency ω_n and the critical damping coefficient c_cr.  [0.5]

Ans: ω_n = √(k/m) = √(300.0/3.0) = √100 = 10.00 rad/s

c_cr = 2√(km) = 2√(300.0 × 3.0) = 2√900 = 2 × 30.00 = 60.00 N·s/m

(c)  Calculate the damping ratio ζ and state the type of damping.  [0.5]

Ans: ζ = c / c_cr = 12.0 / 60.00 = 0.2000

Since ζ = 0.2000 < 1 → the system is UNDERDAMPED.

It will oscillate with exponentially decaying amplitude.

(d)  Calculate the damped natural frequency ω_d.  [0.5]

Ans: ω_d = ω_n √(1 − ζ²) = 10.00 × √(1 − 0.2000²)

= 10.00 × √(1 − 0.040000)

= 10.00 × 0.979796

= 9.7980 rad/s ≈ 9.80 rad/s

(e)  Calculate the logarithmic decrement δ and find the ratio of two successive peak amplitudes.  [0.5]

Ans: δ = 2πζ / √(1 − ζ²) = 2π × 0.2000 / √(1 − 0.040000)

= 1.2566 / 0.979796

= 1.2825

Ratio of successive peaks: x_n / x_(n+1) = e^δ = e^1.2825 = 3.6058

Or equivalently x_(n+1) / x_n = e^(−δ) = 0.2773

Each successive peak is ≈ 27.73% of the previous one.

(f)  If the damper is removed (c = 0), write the displacement as a function of time x(t) for the given initial conditions.  [0.5]

Ans: With c = 0 (undamped), EOM: mẍ + kx = 0

x(t) = x₀ cos(ω_n t) = 0.05 cos(10.00 t)  m

(Since initial conditions are x(0) = 0.05 m, ẋ(0) = 0, the solution is a pure cosine with amplitude 0.05 m and period T = 2π/10.00 = 0.6283 s)

— End of Set 1 Model Answers —