Mukesh Patel School of Technology Management & Engineering

B.Tech Mechatronics Engineering, Semester IV

Re-Examination — MODEL ANSWERS — Set 2

| Course: | Dynamic Systems Modeling (702MH0C023) | Date: |  |
| --- | --- | --- | --- |
| Duration: | 45 minutes | Total Marks: | 10 |

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

Q1(a) [2 Marks]    Bloom: BL 1–2 (Remember / Understand)  |  CO1

Answer the following:

(a)  Write the expressions for the radial and transverse components of acceleration in polar coordinates (a_r and a_θ) and identify the centripetal term.  [0.5]

Ans: In polar coordinates the acceleration components are:

a_r = r̈ − rθ̇²  (radial component)

a_θ = rθ̈ + 2ṙθ̇  (transverse component)

The term −rθ̇² in a_r is the centripetal acceleration, directed radially inward. The term 2ṙθ̇ in a_θ is the Coriolis component arising from simultaneous radial and angular motion.

(b)  State Hamilton's principle (principle of least action) and explain its relationship to the Euler-Lagrange equation.  [0.5]

Ans: Hamilton's principle states that the actual path taken by a mechanical system between two time instants t₁ and t₂ is the one that makes the action integral stationary:

δS = δ ∫(t₁ to t₂) L dt = 0, where L = T − V.

Applying the calculus of variations to this condition yields the Euler-Lagrange equation:

d/dt(∂L/∂q̇) − ∂L/∂q = 0.

(c)  Differentiate between holonomic and non-holonomic constraints. Give one example of each from a mechanical system.  [0.5]

Ans: Holonomic constraint: can be expressed as f(q₁, q₂, …, t) = 0 (depends only on coordinates and time, not velocities).

Example: A bead on a wire — its position is restricted to the wire curve.

Non-holonomic constraint: involves velocities or inequalities and cannot be reduced to a coordinate-only equation.

Example: A wheel rolling without slipping — the constraint dx = R dθ relates velocity components but cannot be integrated into a position-only equation for all paths.

(d)  State the parallel axis theorem for moment of inertia and give its mathematical expression.  [0.5]

Ans: The parallel axis theorem states that the moment of inertia about any axis parallel to one through the center of mass is:

I = I_cm + md²

where I_cm = moment of inertia about the CM axis, m = total mass, d = perpendicular distance between the two axes. I is always smallest about the CM axis.

Q1(b) [2 Marks]    Bloom: BL 2–3 (Understand / Apply)  |  CO1, CO2

(a)  A particle moves along a spiral with r = 3 m, ṙ = 0, and θ̇ = 2 rad/s (constant). Calculate the centripetal acceleration and state its direction.  [0.5]

Ans: For circular motion with r = 3 m, ṙ = 0, θ̇ = 2 rad/s:

a_centripetal = rθ̇² = 3 × 2² = 3 × 4 = 12 m/s²

(In polar form: a_r = r̈ − rθ̇² = 0 − 12 = −12 m/s²)

Direction: radially inward, toward the center of curvature.

(b)  Explain the concept of generalized coordinates. For a double pendulum in a plane, state the number of DOF and identify the generalized coordinates.  [0.5]

Ans: Generalized coordinates are the minimum set of independent coordinates needed to completely describe a system's configuration. They need not be Cartesian — they can be angles, arc lengths, etc.

A double pendulum in a plane has 2 DOF.

Generalized coordinates: θ₁ (angle of first link) and θ₂ (angle of second link), both measured from the vertical.

(c)  Define the Coriolis acceleration. State the expression for its magnitude and explain in which physical situations it becomes significant.  [0.5]

Ans: The Coriolis acceleration arises when an object moves with velocity v_rel in a reference frame rotating at angular velocity Ω:

a_Coriolis = 2Ω × v_rel,  magnitude = 2Ωv_rel

It is perpendicular to both Ω and v_rel. It becomes significant in large-scale meteorology (weather systems), long-range ballistics, and fast-rotating machinery where the product Ωv_rel is comparable to other acceleration terms.

(d)  A uniform rod of mass m = 3 kg and length L = 1.2 m is pivoted at one end. Calculate its moment of inertia about the pivot using the parallel axis theorem.  [0.5]

Ans: For a uniform rod: I_cm = (1/12)mL² = (1/12) × 3 × 1.2² = (1/12) × 3 × 1.44 = 0.36 kg·m².

Parallel axis theorem: I_pivot = I_cm + m(L/2)²

= 0.36 + 3 × (0.6)² = 0.36 + 3 × 0.36 = 0.36 + 1.08

= 1.44 kg·m².

(Check: I_pivot = (1/3)mL² = (1/3) × 3 × 1.44 = 1.44 kg·m² ✓)

Solve any TWO from Q2, Q3, Q4  [3 Marks each = 6 Marks]

Q2. [3 Marks]    Bloom: BL 3 (Apply)  |  CO1, CO2

A simple pendulum consists of a point mass m = 2 kg attached to a massless, inextensible string of length L = 0.5 m. The bob swings in a vertical plane under gravity (g = 9.81 m/s²). Using the angle θ from the vertical as the generalized coordinate, analyze the system using the Lagrangian method.
Use the following relations: Lagrangian L = T − V; Euler-Lagrange equation: d/dt(∂L/∂q̇) − ∂L/∂q = 0; for small angles: sin θ ≈ θ.

(a)  State the number of degrees of freedom of this system and identify the generalized coordinate.  [0.5]

Ans: The pendulum has 1 degree of freedom (DOF = 1).

Generalized coordinate: θ — the angle of the string measured from the vertical.

(b)  Write the position of the mass in Cartesian coordinates (x, y) in terms of θ and L.  [0.5]

Ans: Taking the pivot as the origin, with x horizontal and y vertically downward:

x = L sin θ = 0.5 sin θ

y = L cos θ = 0.5 cos θ

Differentiating: ẋ = Lθ̇ cos θ,  ẏ = −Lθ̇ sin θ

(c)  Derive the kinetic energy T in terms of θ, θ̇, m, and L.  [0.5]

Ans: v² = ẋ² + ẏ² = L²θ̇²cos²θ + L²θ̇²sin²θ = L²θ̇²

T = ½mv² = ½ × 2.0 × 0.5² × θ̇² = ½mL²θ̇²

T = ½(2.0)(0.5²)θ̇² = 0.25 θ̇²  J

(d)  Write the potential energy V in terms of θ, m, g, and L.  [0.5]

Ans: Taking the pivot as the datum (y positive downward):

V = −mgy = −mgL cos θ = −2.0 × 9.81 × 0.5 × cos θ

V = −9.81 cos θ  J

(e)  Form the Lagrangian L = T − V and apply the Euler-Lagrange equation to derive the equation of motion.  [0.5]

Ans: L = T − V = ½mL²θ̇² + mgL cos θ

Euler-Lagrange: d/dt(∂L/∂θ̇) − ∂L/∂θ = 0

∂L/∂θ̇ = mL²θ̇ = 2.0 × 0.5² × θ̇ = 0.5 θ̇

d/dt(mL²θ̇) = mL²θ̈ = 0.5 θ̈

∂L/∂θ = −mgL sin θ = −9.81 sin θ

EOM: 0.5 θ̈ + 9.81 sin θ = 0

Simplifying (divide by mL²): θ̈ + (g/L) sin θ = 0

→ θ̈ + (9.81/0.5) sin θ = 0  →  θ̈ + 19.62 sin θ = 0

(f)  For small oscillations (sin θ ≈ θ), determine the natural frequency of the pendulum.  [0.5]

Ans: For small angles, sin θ ≈ θ → θ̈ + (g/L)θ = 0

This is SHM with ω_n² = g/L = 9.81/0.5 = 19.62

ω_n = √(19.62) = 4.429 rad/s

Time period T = 2π/ω_n = 2π/4.429 = 1.419 s

Q3. [3 Marks]    Bloom: BL 3–4 (Apply / Analyze)  |  CO2

A horizontal turntable rotates at a constant angular velocity Ω = 5 rad/s about a vertical axis. A small ball at r = 2 m from the center is pushed radially outward at a constant relative speed v_rel = 3 m/s (ṙ = 3 m/s, r̈ = 0). The angular acceleration of the turntable is zero (α = 0).

(a)  Write the general expression relating the acceleration observed from the fixed (inertial) frame to quantities measured in the rotating frame. Identify each term.  [0.5]

Ans: The fixed-frame (inertial) acceleration, expressed in terms of rotating-frame quantities, is:

a⃗_fixed = a⃗_rel + 2Ω⃗ × v⃗_rel + α⃗ × r⃗ + Ω⃗ × (Ω⃗ × r⃗)

where:

a⃗_rel = acceleration of the object in the rotating frame (r̈ = 0 here),

2Ω⃗ × v⃗_rel = Coriolis acceleration,

α⃗ × r⃗ = Euler (angular acceleration) term (zero here since α = 0),

Ω⃗ × (Ω⃗ × r⃗) = centripetal acceleration (directed radially inward).

(b)  Calculate the centripetal acceleration magnitude at r = 2 m.  [0.5]

Ans: a_centripetal = Ω²r = 5.0² × 2.0 = 25 × 2.0 = 50.0 m/s²

Direction: radially inward (toward the axis of rotation).

(c)  Calculate the Coriolis acceleration magnitude.  [0.5]

Ans: a_Coriolis = 2Ωv_rel = 2 × 5.0 × 3.0 = 30.0 m/s²

(d)  State the direction of the Coriolis acceleration relative to the radial velocity.  [0.5]

Ans: By the right-hand rule for 2Ω⃗ × v⃗_rel (Ω along vertical, v_rel radially outward): the Coriolis acceleration is perpendicular to v_rel, directed tangentially in the direction opposite to the platform's rotation.

(e)  Calculate the total acceleration magnitude of the object as observed from the fixed (inertial) frame.  [0.5]

Ans: |a_total| = √(a_cent² + a_Cor²) = √(50.0² + 30.0²)

= √(2500 + 900) = √3400 = 58.31 m/s²

(f)  If the ball continues at the same radial speed, find the centripetal acceleration when r = 4 m and comment on how it changes.  [0.5]

Ans: At r = 4 m: a_centripetal = Ω²r = 5.0² × 4 = 100.0 m/s²

Compared to 50.0 m/s² at r = 2.0 m, the centripetal acceleration has doubled. It increases linearly with radial distance (a_cent ∝ r), while the Coriolis acceleration (30.0 m/s²) remains unchanged since v_rel is constant.

Q4. [3 Marks]    Bloom: BL 4–5 (Analyze / Evaluate)  |  CO2, CO3

A uniform rod of mass m = 2 kg and length L = 1.2 m is pivoted at one end and oscillates as a physical (compound) pendulum under gravity. Take g = 9.81 m/s².
Use: I_rod_cm = (1/12)mL²; parallel axis theorem: I = I_cm + md²; period of physical pendulum: T = 2π√(I_pivot / (mgd)), where d = distance from pivot to center of mass.

(a)  Calculate the moment of inertia of the rod about its center of mass (I_cm).  [0.5]

Ans: I_cm = (1/12)mL² = (1/12) × 2.0 × 1.2²

= (1/12) × 2.0 × 1.44 = 0.2400 kg·m²

(b)  Using the parallel axis theorem, calculate the moment of inertia about the pivot at one end (I_pivot).  [0.5]

Ans: I_pivot = I_cm + m(L/2)² = 0.2400 + 2.0 × (1.2/2)²

= 0.2400 + 2.0 × 0.6² = 0.2400 + 0.7200

= 0.9600 kg·m²

(Check: I_pivot = (1/3)mL² = (1/3) × 2.0 × 1.44 = 0.9600 kg·m² ✓)

(c)  Find the distance d of the center of mass from the pivot.  [0.5]

Ans: d = L/2 = 1.2/2 = 0.6 m

(The center of mass of a uniform rod is at its midpoint.)

(d)  Write the equation of motion for small-angle oscillations and determine the natural frequency ω_n.  [0.5]

Ans: Restoring torque: τ = −mgd sin θ ≈ −mgdθ  (small angle)

Equation of motion: I_pivot θ̈ + mgd θ = 0

→ θ̈ + (mgd / I_pivot) θ = 0

ω_n = √(mgd / I_pivot) = √(2.0 × 9.81 × 0.6 / 0.9600)

= √(11.7720 / 0.9600) = √12.2625

= 3.5018 rad/s ≈ 3.502 rad/s

(e)  Calculate the time period of oscillation T.  [0.5]

Ans: T = 2π / ω_n = 2π / 3.5018 = 1.7943 s ≈ 1.794 s

(f)  Compare this period with that of a simple pendulum of the same length (L = 1.2 m) and explain the difference.  [0.5]

Ans: Simple pendulum of same length L = 1.2 m:

T_simple = 2π√(L/g) = 2π√(1.2/9.81) = 2π × 0.3497

= 2.1975 s ≈ 2.198 s

Physical pendulum period (1.794 s) < Simple pendulum period (2.198 s).

This is because the physical pendulum's mass is distributed along its length, with some mass closer to the pivot than L. The effective pendulum length L_eff = I_pivot/(md) = 0.9600/(2.0×0.6) = 0.8000 m < 1.2 m.

— End of Set 2 Model Answers —